Required Information List for CV Beta Patch

(Internal Use)

- MPSA ID: MPSA-191101

- Affected Product: AWK-3121

- Affected Firmware: v1.14 and versions prior

- Vulnerability: CVE-2018-10690, CVE-2018-10691, CVE-2018-10692, CVE-2018-10693, CVE-2018-10694, CVE-2018-10695, CVE-2018-10696, CVE-2018-10697, CVE-2018-10698, CVE-2018-10699, CVE-2018-10700, CVE-2018-10701, CVE-2018-10702, CVE-2018-10703.

- Impact: Unauthorized personnel can acquire Admin privilege. System could be compromised via command injection attack. Communication data can be sniffed and viewed by unintended audience. Refer to advisory note for detailed table.

- Root cause of vulnerability: Using unencrypted protocol in default configuration. Design weakness allowing command injection attack in several input fields. Unsecure cookie used vulnerable to extract sensitive login credentials. File server allow download of sensitive file without authentication.

Detailed root cause mapping Table

| Item | Vulnerability Reference | Root Cause |
| --- | --- | --- |
| 1 | CVE-2018-10690 | The device by default allows HTTP traffic thus providing an insecure communication mechanism for a user connecting to the web server. |
| 2 | CVE-2018-10691 | Vulnerable to unauthorized systemlog.log download. |
| 3 | CVE-2018-10692 | Vulnerable to cross-site scripting attack to steal the cookie. |
| 4 | CVE-2018-10693 | The POST parameter "srvName" is susceptible to a buffer overflow. |
| 5 | CVE-2018-10694 | The device provides a Wi-Fi connection that is open and does not use any encryption mechanism by default. |
| 6 | CVE-2018-10695 | The POST parameters "to1,to2,to3,to4" are all susceptible to buffer overflow. |
| 7 | CVE-2018-10696 | Web interface is not protected against CSRF attacks |
| 8 | CVE-2018-10697 | The POST parameter "srvName" is susceptible to command injection. |
| 9 | CVE-2018-10698 | The device enables an unencrypted TELNET service by default. |
| 10 | CVE-2018-10699 | The POST parameter "iw_privatePass" is susceptible to command injection. |
| 11 | CVE-2018-10700 | The POST parameter "iw_board_deviceName" is susceptible to XSS payload injection |
| 12 | CVE-2018-10701 | The POST parameter "iw_filename" is susceptible to buffer overflow. |
| 13 | CVE-2018-10702 | The POST parameter "iw_filename" is susceptible to command injection via shell metacharacters. |
| 14 | CVE-2018-10703 | The POST parameter "iw_serverip" is susceptible to buffer overflow. |

- How did SBU verify this patch? SQA verified test case with pass report.

- Any other model are affected? Yes, expect old products prior to AWK-3121 affected, all phased out.

- Any site effect? No.

- Next VR schedule: No VR schedule, this is phased out product.

FAQ (External Use)

1. The safety issues only reported in these models, are there any other models also have the same safety issue? Expect old models prior to AWK-3121 have same issues, but phased out.

2. Will these affected models have VR in the future? No, all phased out. P.S. SBU suggest to handle only per request basis.

3. Directly firmware upgrade? Or any specific upgrade ways? Direct FW upgrade via web console.